Contents

1.	OSI and TCP Layer	2

2.	Ethernet Frame: -	4

2.1.	Ethernet frame structure: -	4

3.	Hub and Switch	6

3.1.	Hub: -	6

3.2.	Switch: -	6

4.	IEEE802.1Q tagged frame and VLAN Tag	7

5.	Virtual LAN (VLAN)	8

5.1.	Advantages of VLANs	8

5.2.	Trunking	9

5.3.	VLAN types	9

6.	TCP/IP: -	10

6.1.	Internet Protocol: -	10

IPv4 Header Components/Fields: -	11

6.2.	TCP segment: -	13

6.3.	IP Address: -	13

6.3.1.	Tricks to remember 5 classes of IPV4 address: -	15

7.	MII( Media Independent Interface)	16

7.1.	Standard MII: -	16

Transmitter signals	17

Receiver signals	18

Management signals :	19

# OSI and TCP Layer

Figure - 1

Figure – 2

Figure – 3

# Ethernet Frame: -

Standard: -

IEEE 802.3 - Ethernet

IEEE 802.11 – Wi-Fi

## Ethernet frame structure: -

Ethernet frame Normal IEEE 802.3 compliant Ethernet frames are between 64 and 1518 bytes long. They are made up of five or six different fields:

- destination MAC address

- source MAC address

- Type/length field

- Data payload

- Optional Padding field

- CRC

- Additionally, when transmitted on the Ethernet medium, a 7-byte preamble field and Start-Of-Frame (SOF) delimiter bytes are appended.

| Preamble | SOF Delimiter | Destination address (such as Multicast, Broadcast and Unicast) | Source address | Type/Length | DATA/ Payload (with optional padding) | CRC |
| --- | --- | --- | --- | --- | --- | --- |

Preamble: -

Preamble is a series of 56 bits alternating with 1 and 0. These bits are used for synchronization

and give each participant the time to observe the activity on the bus before the actual data arrives.

SFD: -

The start of frame delimiter (10101011), the last byte of the preamble, indicates to the receiver that the actual data is on its way.

MAC address: -

Size of MAC address: 6 bytes (48 bits). The MAC broadcast address is FF FF FF FF FF FF.

TYPE: -

The TYPE field is different for IEEE802.3 and Ethernet II (DIX standard).

For Ethernet II, the field type refers to the higher-level protocol that uses an Ethernet frame to send data. Xerox assigns a code of 2 bytes to every protocol that is developed for Ethernet. Some examples:

0600h XNS

0800h IP (Internet Protocol)

0806h ARP protocol

0835h Reverse ARP protocol

8100h IEEE802 1.q tag frame (VLAN)

The IEEE802.3 defines the field TYPE as LENGTH field in order to be able to send the number of actual data bytes.

DATA: -

The data field contains the data to be sent. This data field is transparent- this means that the content of this field is completely free for Ethernet. Only the length has to be a minimum of 46 bytes and not more than 1500 bytes.

PAD: -

The padding bits are random data bits that, if necessary, can be added to the data in order to reach the minimum required 46 bytes.

FCS: -

The check sum is a 4-byte CRC value that the sender creates and sends. The receiver can check the integrity of the data with this code.

Note: -

Ping: - ICMP is used for testing network connections using ping program.

# Hub and Switch

## Hub: -

A hub is actually a multiport repeater: it regenerates incoming signals to all other ports as can be seen from figure below.

Figure – 4

## Switch: -

Switch is used to interlink LAN segments with more intelligence. Before a packet is transmitted from one segment to the other segment via a switch, it checks the MAC address and on this basis it decides whether to transport to the other segment or not. Switch in accordance with OSI model is shown below.

Figure – 5

A MAC address table is updated from a software point of view for every port. This table is filled by listening to the relevant segment of the network and by copying all MAC addresses that occur on that segment to the table. Every address is retained for a limited time and is deleted again as soon as a certain time (the hold time) has lapsed. This technique avoids that inactive stations are addressed or that stations are not recognized anymore.

Linking the segments of a local network via a switch has a number of advantages over the link with a repeater or a hub. When using a switch, a segment is not loaded with the frames of the other segment that do not belong there from an addressing point of view. The load per segment is reduced by this bridge function. At the same time, fault situations are not transmitted as the switch also checks the correct building of the frame.

# IEEE802.1Q tagged frame and VLAN Tag

The IEEE802.1Q describes 4 extra bytes, divided into two extra fields in the Ethernet frame in

order to use for new applications. One of these applications is VLAN.

Figure – 6

Description of the extra fields:

- TYPE(TAG), 2 bytes: has the value 8100h to specify that this frame is a tagged frame

and therefore contains an extra information field

- VLAN TAG/TPID, 2 bytes: VLAN Tag Protocol Identifier

– User priority, 3 bits:

The priority of the frame is included; the priority code (a number between 0 and 7) is described in IEEE802.1p.

– CFI: Canonical Format Indicator.

The IEEE802.1Q is only developed for Ethernet or Token Ring. This bit is 0 for Ethernet and 1 for Token Ring.

– VLAN ID: Identification of the VLAN, 4094 possibilities.

FFFFh reserved

0000h no VLAN, frames with priority (Profinet IO)

# Virtual LAN (VLAN)

A VLAN or Virtual Local Area Network is a group of participants in a large network that form

a separate network in a logic manner. This means that several logic groups can be created

on a large physical network. A VLAN has an own broadcast domain. Data packets are only

transmitted within a VLAN. The participants may physically be far away from each other but

have to be on one and the same physical network. Some examples of the organization of a

network:

- By department: one VLAN for Sales, another VLAN for Engineering and another VLAN

for Automation.

- By hierarchy: one VLAN for management, another VLAN for managers and another

VLAN for employees.

- By use: one VLAN for users that require e-mail and another for multimedia users.

## Advantages of VLANs

The greatest advantage of VLANs is the segmentation of the network. Other examples are

additional security and restriction on network load.

- Moving devices around: it is easier to move devices around in the network in a traditional network, cabling has to be changed when a user moves from one subnet to another. The relocation from one VLAN to another does not require change in cabling. It only requires a setting on the switch. A station from Sales can, for example, be moved to a network connection that belongs to Engineering. The port has to be set up as a member of the VLAN Engineering but does not require new cabling.

- Additional security: devices of a VLAN can only communicate with devices in the same VLAN. If a device of VLAN Sales wants to communicate with the VLAN Automation, then this connection has to be set up in a router.

- Restriction on network traffic: for a traditional network, broadcasts can cause an overloaded network. Broadcast messages are often sent to devices that do not need these messages. VLANs limit this problem as a broadcast message from one VLAN is not sent to the other VLAN.

## Trunking

Trunking is a method to send data from different VLANs between two switches. Only one port

per device is required for this. There are different ways of trunking.

- ISL: InterSwitch Link, this is a widely used proprietary protocol of Cisco

- IEEE802.1Q: this is a standard that is supported by several switch manufacturers.

For trunking, a piece of code (tag) is added that states which VLAN the sent package comes

from. Thanks to this system, the benefits of VLAN are retained. A router is needed to route data traffic between the different VLANs.

## VLAN types

The different types of VLANs can be divided into two types: static and dynamic VLANs.

- Static VLANs :

Static VLANs are port-based. Depending on the port of a switch to which a user connects, this belongs to one or the other VLAN.

Advantages:

– Easy to configure

– Everything is done using the switch. The user hardly notices anything

Disadvantages:

– If a user connects his PC to the wrong port, then the administrator has to do a

reconfiguration.

– If a second switch is connected to a port that belongs to a certain VLAN, then 
  all computers that one connects to this switch will automatically belong to this    
  VLAN.

- Dynamic VLANs :

Dynamic VLANs are not based on ports of a switch but on the address of the user or the used protocol.

Advantage:

Everyone can connect his computer to any port and still be part of the correct VLAN.

Disadvantage:

The cost of this VLAN type is higher as it requires special hardware.

# TCP/IP: -

Figure - 7

Figure – 7

## Internet Protocol: -

The most important features of the IP protocol are:

- Routing of a data packet over the Internet. Every host is identified by a 32-bit IP address.

- It is a connectionless protocol. Every packet can follow a different route to the same

target host when sending different IP packets. No fixed physical connection is     created.

- The IP header is at least 20 bytes long. When using the options field, the header can be 60 bytes maximum. A header check sum is created.

- The Internet Protocol (IP) is applicable to the network level (layer 3 of the OSI model). This layer is responsible for the presentation and transportation of information over different networks.

Uniform addressing is required to realize this: the IP address.

- Internet Protocol being a layer-3 protocol (OSI) takes data Segments from layer-4 (Transport) and divides it into packets. IP packet encapsulates data unit received from above layer and add to its own header information.

| IP Header | Layer – 4 Data |
| --- | --- |

Figure – 8: IP Packet

The encapsulated data is referred to as IP Payload. IP header contains all the necessary information to deliver the packet at the other end.

## IPv4 Header Components/Fields: -

Figure – 9: IP Header Format

Following are various components/fields of IP packet header

- Version: The first IP header field is a 4-bit version indicator. In IPv4, the value of its four bits is set to 0100, which indicates 4 in binary. However, if the router does not support the specified version, this packet will be dropped.

- Internet Header Length: Internet header length, shortly known as IHL, is 4 bits in size. It is also called HELEN (Header Length). This IP component is used to show how many 32-bit words are present in the header.

- Type of Service: Type of Service is also called Differentiated Services Code Point or DSCP. This field is provided features related to the quality of service for data streaming or VoIP calls. The first 3 bits are the priority bits. It is also used for specifying how you can handle Datagram.

- Total length: The total length is measured in bytes. The minimum size of an IP datagram is 20 bytes and the maximum, it can be 65535 bytes. HELEN and Total length can be used to calculate the dimension of the payload. All hosts are required to be able to read 576-byte data-grams. However, if a datagram is too large for the hosts in the network, the fragmentation method is widely used.

- Identification: Identification is a packet that is used to identify fragments of an IP datagram uniquely. Some have recommended using this field for other things like adding information for packet tracing, etc.

- IP Flags: Flag is a three-bit field that helps you to control and identify fragments. The following can be their possible configuration:

Bit 0: is reserved and has to be set to zero

Bit 1: means do not fragment

Bit 2: means more fragments.

- Fragment Offset: Fragment Offset represents the number of Data Bytes ahead of the particular fragment in the specific Datagram. It is specified in terms of the number of 8 bytes, which has a maximum value of 65,528 bytes.

- Time to live: It is an 8-bit field that indicates the maximum time the Datagram will be live in the internet system. The time duration is measured in seconds, and when the value of TTL is zero, the Datagram will be erased. Every time a datagram is processed its TTL value is decreased by one second. TTL are used so that datagrams are not delivered and discarded automatically. The value of TTL can be 0 to 255.

- Protocol: This IPv4 header is reserved to denote that internet protocol is used in the latter portion of the Datagram. For Example, 6 number digit is mostly used to indicate TCP, and 17 is used to denote the UDP protocol.

- Header Checksum: The next component is a 16 bits header checksum field, which is used to check the header for any errors. The IP header is compared to the value of its checksum. When the header checksum is not matching, then the packet will be discarded.

- Source Address: The source address is a 32-bit address of the source used for the IPv4 packet.

- Destination address: The destination address is also 32 bit in size stores the address of the receiver.

- IP Options: It is an optional field of IPv4 header used when the value of IHL (Internet Header Length) is set to greater than 5. It contains values and settings related with security, record route and time stamp, etc. You can see that list of options component ends with an End of Options or EOL in most cases.

- Data: This field stores the data from the protocol layer, which has handed over the data to the IP layer.

## TCP segment: -

Transmission Control Protocol accepts data from a data stream, divides it into chunks, and adds a TCP header creating a TCP segment. The TCP segment is then encapsulated into an Internet Protocol (IP) datagram, and exchanged with peers.

A TCP segment consists of a segment header and a data section. The segment header contains 10 mandatory fields, and an optional extension field (Options, pink background in table). The data section follows the header and is the payload data carried for the application. The length of the data section is not specified in the segment header; it can be calculated by subtracting the combined length of the segment header and IP header from the total IP datagram length specified in the IP header.

| TCP Segment Header | Data |
| --- | --- |

Figure – 10: TCP segment

Figure – 11: TCP Segment Header

Fields in TCP segment header are described in below web page.

https://en.wikipedia.org/wiki/Transmission_Control_Protocol#TCP_segment_structure

## IP Address: -

IP address in IPv4 is of 32 bits or 4 bytes. IP address consists of both Network ID and Host ID.

| Net ID | Host ID |
| --- | --- |

Figure – 12: IP address

IP addresses are divided into 5 different classes: -

Class A, Class B, Class C, Class D, Class E.

Class D is added in order to send multicast messages in a simple way. Class E currently has no function yet.

| Byte 1 | Byte 2 | Byte 3 | Byte 4 |
| --- | --- | --- | --- |

Figure – 13: IP address bytes

Network ID and Host ID in different IP address classes are as shown below.

Figure – 14: Bytes allocated for Net ID and Host ID in different IP address classes

Note: -

0.0.0.0 - default routing

127.0.0.0 - loopback testing

255.255.255.255 - broadcast address

### Tricks to remember 5 classes of IPV4 address: -

The class of an IP address is determined by the first octet.

Bits in a byte can be numbered as:

128 – 64 – 32 – 16 – 8 – 4 – 2 – 1

Figure – 15

Class A IP address has the first octet starting from 0.

Class B starts from 0+128 = 128.

Class C starts from 0+128+64=192.

Class D starts from 0+128+64+32=224.

Class E starts from 0+128+64+32+16=240.

Similarly, in binary form Most Significant Bits in first octet starts as below depending on the class.

Class A starts with bit 0 in first Octet.

Class B – 10

Class C – 110

Class D – 1110

Class e - 1111

[Done till page – 46 of Ethernet_Basic pdf and 30:00 of KM-44 level-2 video]

# MII( Media Independent Interface)

Reference: https://en.wikipedia.org/wiki/Media-independent_interface

The media-independent interface (MII) defines the interface between the MAC and the PHY.

MII is standardized by IEEE 802.3u and connects different types of PHYs to MACs.

The original MII transfers network data using 4-bit nibbles in each direction (4 transmit data bits, 4 receive data bits). The data is clocked at 25 MHz to achieve 100 Mbit/s throughput. The original MII design has been extended to support reduced signals and increased speeds. Current variants include:

- Reduced media-independent interface (RMII)

- Gigabit media-independent interface (GMII)

- Reduced gigabit media-independent interface (RGMII)

- Serial media-independent interface (SMII)

- Serial gigabit media-independent interface (serial GMII, SGMII)

- High serial gigabit media-independent interface (HSGMII)

- Quad serial gigabit media-independent interface (QSGMII)

- 10-gigabit media-independent interface (XGMII)

MDIO: -

The Management Data Input/Output (MDIO) serial bus is a subset of the MII that is used to transfer management information between MAC and PHY. At power up, using autonegotiation, the PHY usually adapts to whatever it is connected to unless settings are altered via the MDIO interface.

## Standard MII: -

The standard MII features a small set of registers:

- Basic Mode Configuration (#0)

- Status Word (#1)

- PHY Identifier (#2, #3)

- Auto-Negotiation Advertisement (#4)

- Auto-Negotiation Link Partner Base Page Ability (#5)

- Auto-Negotiation Expansion (#6)

- Auto-Negotiation Next Page Transmit (#7)

- Auto-Negotiation Link Partner Received Next Page (#8)

- MASTER-SLAVE Control Register (#9)

- MASTER-SLAVE Status Register (#10)

- PSE Control register (#11)

- PSE Status register (#12)

- MMD Access Control Register (#13)

- MMD Access Address Data Register (#14)

Register #15 is reserved; registers #16 through #31 are vendor-specific. The registers are used to configure the device and to query the current operating mode.

The MII Status Word is the most useful datum, since it may be used to detect whether an Ethernet NIC is connected to a network. It contains a bit field with the following information:

### Transmitter signals

The transmit clock(TX_CLK)  is a free-running clock generated by the PHY based on the link speed (25 MHz for 100 Mbit/s, 2.5 MHz for 10 Mbit/s). The remaining transmit signals are driven by the MAC synchronously on the rising edge of TX_CLK.

This arrangement allows the MAC to operate without having to be aware of the link speed. The transmit enable signal is held high during frame transmission and low when the transmitter is idle.

### Receiver signals

The receive clock is recovered from the incoming signal during frame reception. When no clock can be recovered (i.e. when the medium is silent), the PHY must present a free-running clock as a substitute.

The CRS and COL signals are asynchronous to the receive clock, and are only meaningful in half-duplex mode. Carrier sense is high when transmitting, receiving, or the medium is otherwise sensed as being in use. If a collision is detected, COL also goes high while the collision persists.

In addition, the MAC may weakly pull-up the COL signal, allowing the combination of COL high with CRS low (which a PHY will never produce) to serve as indication of an absent/disconnected PHY.

### Management signals :

MDIO(Management Data Input/Output)

Limitation:

The interface requires 18 signals, out of which only two (MDIO and MDC) can be shared among multiple PHYs. This presents a problem, especially for multiport devices; for example, an eight-port switch using MII would need 8 × 16 + 2 = 130 signals.

# OABR: -

OABR (Open Alliance BroadR-Reach) is a physical transmission technology that was originally developed by Broadcom and standardized as IEEE 100BASE-T1.

BroadR-Reach technology is an Ethernet physical layer standard designed for automotive connectivity applications. BroadR-Reach allows multiple in-vehicle systems to simultaneously access information over unshielded single twisted pair cable. BroadR-Reach was invented and is promoted by Broadcom Corporation, now Broadcom Limited.

## Objectives of OABR PHY: -

- Provide a PHY that supports full duplex operation at 100 Mb/s over one pair unshielded twisted pair (UTP) or better cable for at least 15m.

- Provide compatibility with the MII (IEEE 802.3 Clause 22) and IEEE 802.3 MAC operating at 100 b/s.

- Maintain a bit error ratio (BER) of less than or equal 10^-10 at the MAC interface (over a one pair UTP cabling)

- Support a start-up procedure which enables the time from power_on=FALSE to valid data to be less than 200ms.

- Support 100Mb/s operation in automotive and industrial environments (e.g., EMC, temperature).

# Ethernet PHY: -

The Ethernet PHY is connected to a media access controller (MAC). The MAC controls the data-link-layer portion of the OSI model.